# 1. Test Codes

## a) I2C check

#include <Wire.h>

void setup() {

Wire.begin();

Serial.begin(9600);

while (!Serial);

Serial.println("\nI2C Scanner");

}

void loop() {

byte error, address;

int nDevices;

Serial.println("Scanning...");

nDevices = 0;

for(address = 1; address < 127; address++ ) {

// The i2c_scanner uses the return value of

// the Write.endTransmisstion to see if

// a device did acknowledge to the address.

Wire.beginTransmission(address);

error = Wire.endTransmission();

if (error == 0) {

Serial.print("I2C device found at address 0x");

if (address < 16) Serial.print("0");

Serial.print(address, HEX);

Serial.println("  !");

nDevices++;

}

}

if (nDevices == 0)

Serial.println("No I2C devices found\n");

else

Serial.println("done\n");

delay(5000);           // wait 5 seconds for next scan

}

## b) Motor Test

/*

* ChromoBot - Motor Test

* ----------------------

* 1. Forward (2s)

* 2. Backward (2s)

* 3. Spin Left (2s)

* 4. Spin Right (2s)

* 5. Stop (2s)

*/

// --- MOTOR PINS ---

const int L_PWM = 2; const int L_IN1 = 22; const int L_IN2 = 23;

const int R_PWM = 3; const int R_IN1 = 24; const int R_IN2 = 25;

void setup() {

// Set all motor pins as outputs

pinMode(L_PWM, OUTPUT); pinMode(L_IN1, OUTPUT); pinMode(L_IN2, OUTPUT);

pinMode(R_PWM, OUTPUT); pinMode(R_IN1, OUTPUT); pinMode(R_IN2, OUTPUT);

Serial.begin(9600);

Serial.println("--- MOTOR TEST STARTING ---");

delay(2000);

}

void loop() {

// 1. FORWARD

Serial.println("FORWARD");

move(100, 100);

delay(2000);

stopMotors(); delay(1000);

// 2. BACKWARD

Serial.println("BACKWARD");

move(-100, -100);

delay(2000);

stopMotors(); delay(1000);

// 3. SPIN LEFT

Serial.println("SPIN LEFT");

move(-100, 100);

delay(2000);

stopMotors(); delay(1000);

// 4. SPIN RIGHT

Serial.println("SPIN RIGHT");

move(100, -100);

delay(2000);

stopMotors(); delay(2000);

}

// --- HELPER FUNCTION ---

void move(int l, int r) {

// Left Motor Logic

if(l > 0) {

digitalWrite(L_IN1, LOW); digitalWrite(L_IN2, HIGH); analogWrite(L_PWM, l);

} else {

digitalWrite(L_IN1, HIGH); digitalWrite(L_IN2, LOW); analogWrite(L_PWM, abs(l));

}

// Right Motor Logic

if(r > 0) {

digitalWrite(R_IN1, LOW); digitalWrite(R_IN2, HIGH); analogWrite(R_PWM, r);

} else {

digitalWrite(R_IN1, HIGH); digitalWrite(R_IN2, LOW); analogWrite(R_PWM, abs(r));

}

}

void stopMotors() {

analogWrite(L_PWM, 0); analogWrite(R_PWM, 0);

digitalWrite(L_IN1, LOW); digitalWrite(L_IN2, LOW);

digitalWrite(R_IN1, LOW); digitalWrite(R_IN2, LOW);

}

## c) Ultrasonic sensor

/*

* ChromoBot - 3 Ultrasonic Sensor Test

* ------------------------------------------------

* Upload this to verify all sensors are working.

* Open Serial Monitor at 9600 baud.

*/

// --- PINS ---

const int TRIG_L = 7;   const int ECHO_L = 8;

const int TRIG_MID = 10; const int ECHO_MID = 9;

const int TRIG_R = 11;  const int ECHO_R = 12;

void setup() {

Serial.begin(9600);

pinMode(TRIG_L, OUTPUT); pinMode(ECHO_L, INPUT);

pinMode(TRIG_MID, OUTPUT); pinMode(ECHO_MID, INPUT);

pinMode(TRIG_R, OUTPUT); pinMode(ECHO_R, INPUT);

Serial.println("--- ULTRASONIC TEST STARTED ---");

Serial.println("L = Left | M = Middle | R = Right");

delay(1000);

}

void loop() {

// Read all three

long distL = readDistance(TRIG_L, ECHO_L);

delay(10); // Small pause to prevent interference

long distM = readDistance(TRIG_MID, ECHO_MID);

delay(10);

long distR = readDistance(TRIG_R, ECHO_R);

// Print nicely

Serial.print("L: ");

Serial.print(distL);

Serial.print(" cm  |  M: ");

Serial.print(distM);

Serial.print(" cm  |  R: ");

Serial.print(distR);

Serial.println(" cm");

delay(500); // Update every half second

}

// Helper function

long readDistance(int trig, int echo) {

digitalWrite(trig, LOW); delayMicroseconds(2);

digitalWrite(trig, HIGH); delayMicroseconds(10);

digitalWrite(trig, LOW);

long duration = pulseIn(echo, HIGH, 30000); // 30ms timeout (~5m)

if (duration == 0) return 999; // Return 999 if no echo (out of range/disconnected)

return duration * 0.0343 / 2;

}

## d) IR array truth test

/*

* IR Sensor Truth Test

* --------------------

* READ THIS OUTPUT TO KNOW YOUR LOGIC:

* - Put Sensor on WHITE -> Note the number (0 or 1).

* - Put Sensor on BLACK -> Note the number (0 or 1).

*/

// PINS (Based on your previous code)

const int S1 = 44;

const int S2 = 45;

const int S3 = 46;

const int S4 = 47;

const int S5 = 48;

void setup() {

pinMode(S1, INPUT);

pinMode(S2, INPUT);

pinMode(S3, INPUT);

pinMode(S4, INPUT);

pinMode(S5, INPUT);

Serial.begin(9600);

Serial.println("--- IR SENSOR TEST STARTED ---");

delay(1000);

}

void loop() {

// Read all sensors

int v1 = digitalRead(S1);

int v2 = digitalRead(S2);

int v3 = digitalRead(S3);

int v4 = digitalRead(S4);

int v5 = digitalRead(S5);

// Print to Serial Monitor

Serial.print("S1:"); Serial.print(v1); Serial.print("  ");

Serial.print("S2:"); Serial.print(v2); Serial.print("  ");

Serial.print("S3:"); Serial.print(v3); Serial.print("  ");

Serial.print("S4:"); Serial.print(v4); Serial.print("  ");

Serial.print("S5:"); Serial.println(v5);

delay(200); // Slow down so you can read it

}

# 2.Line following working code

/*

* ChromoBot - Final Logic (Longer Alignment Fix)

* ------------------------------------------------

* SPEEDS: Straight = 60 | Rotate = 100

* ALIGN TIME: Increased to 1200ms for 26cm robot length.

* * 1. BEND: Stop 1s -> Fwd 60 (1.2s) -> Stop 1s -> Rotate -> Wait for Mid 3.

* 2. DRIFT: Stop 1s -> Rotate [Limit] -> Wait for Mid 3.

* 3. NORMAL: PID Speed 60.

*/

// --- PIN DEFINITIONS ---

const int L_PWM = 2; const int L_IN1 = 22; const int L_IN2 = 23;

const int R_PWM = 3; const int R_IN1 = 24; const int R_IN2 = 25;

const int S1 = 44; const int S2 = 45; const int S3 = 46; const int S4 = 47; const int S5 = 48;

// --- TUNING (UPDATED) ---

#define PID_SPEED   60

#define ALIGN_SPEED 60

#define TURN_SPEED  100

#define DRIFT_LIMIT 400     // Max time to rotate for drift (ms)

// INCREASED THIS TO 1200 (1.2 Seconds)

#define ALIGN_TIME  1200

// --- LOGIC MAP ---

#define LINE 1   // White

#define FLOOR 0  // Black

int lastError = 0;

bool isRunning = true;

#define KP 18.0

#define KD 22.0

void setup() {

pinMode(L_PWM, OUTPUT); pinMode(L_IN1, OUTPUT); pinMode(L_IN2, OUTPUT);

pinMode(R_PWM, OUTPUT); pinMode(R_IN1, OUTPUT); pinMode(R_IN2, OUTPUT);

pinMode(S1, INPUT); pinMode(S2, INPUT); pinMode(S3, INPUT); pinMode(S4, INPUT); pinMode(S5, INPUT);

Serial.begin(9600);

Serial.println("--- READY: 1.2s Align Time ---");

delay(2000);

}

void loop() {

if (!isRunning) return;

int s1 = digitalRead(S1);

int s2 = digitalRead(S2);

int s3 = digitalRead(S3);

int s4 = digitalRead(S4);

int s5 = digitalRead(S5);

int activeSensors = s1 + s2 + s3 + s4 + s5;

// CHECK FOR LINE LOSS (Pink Section)

if (activeSensors == 0) {

delay(100); // 100ms Debounce

if (digitalRead(S3) == FLOOR) {

stopMotors();

isRunning = false;

return;

}

}

// ==========================================

// PRIORITY 1: BEND DETECTION (4 or 5 Sensors)

// ==========================================

if (activeSensors >= 4) {

if (s1 == LINE && s2 == LINE && s3 == LINE && s4 == LINE) {

handleBend(-1); // Left Bend

return;

}

if (s2 == LINE && s3 == LINE && s4 == LINE && s5 == LINE) {

handleBend(1);  // Right Bend

return;

}

}

// ==========================================

// PRIORITY 2: DRIFT DETECTION (3 Sensors)

// ==========================================

if (activeSensors == 3) {

if (s1 == LINE && s2 == LINE && s3 == LINE) {

handleDrift(-1); // Left Drift

return;

}

if (s3 == LINE && s4 == LINE && s5 == LINE) {

handleDrift(1); // Right Drift

return;

}

}

// ==========================================

// PRIORITY 3: NORMAL PID

// ==========================================

if (activeSensors > 0) {

int error = (s1 * -4) + (s2 * -2) + (s3 * 0) + (s4 * 2) + (s5 * 4);

error = error / activeSensors;

int P = error;

int D = error - lastError;

int correction = (KP * P) + (KD * D);

lastError = error;

move(PID_SPEED + correction, PID_SPEED - correction);

}

}

// --- MANEUVERS ---

void handleBend(int direction) {

// 1. Stop 1s

stopMotors(); delay(1000);

// 2. Blind Forward (1200ms now)

// Main loop is paused here, so it ignores black floor.

move(ALIGN_SPEED, ALIGN_SPEED);

delay(ALIGN_TIME);

// 3. Stop 1s

stopMotors(); delay(1000);

// 4. Rotate until Center Found

spinUntilMiddle3(direction, 6000);

}

void handleDrift(int direction) {

stopMotors(); delay(1000);

spinUntilMiddle3(direction, DRIFT_LIMIT);

}

void spinUntilMiddle3(int direction, int timeoutMs) {

unsigned long startTime = millis();

// SPIN LOGIC (Speed 100)

if (direction == -1) {

// Spin Left (Left Back, Right Fwd)

digitalWrite(L_IN1, LOW); digitalWrite(L_IN2, HIGH); analogWrite(L_PWM, TURN_SPEED);

digitalWrite(R_IN1, HIGH); digitalWrite(R_IN2, LOW); analogWrite(R_PWM, TURN_SPEED);

} else {

// Spin Right

digitalWrite(L_IN1, HIGH); digitalWrite(L_IN2, LOW); analogWrite(L_PWM, TURN_SPEED);

digitalWrite(R_IN1, LOW); digitalWrite(R_IN2, HIGH); analogWrite(R_PWM, TURN_SPEED);

}

delay(100); // Clear state

while (millis() - startTime < timeoutMs) {

int s2 = digitalRead(S2);

int s3 = digitalRead(S3);

int s4 = digitalRead(S4);

// Target: Middle 3 White

if (s2 == LINE && s3 == LINE && s4 == LINE) {

stopMotors();

delay(200);

return;

}

}

stopMotors();

}

// --- MOTORS ---

void move(int leftSpeed, int rightSpeed) {

leftSpeed = constrain(leftSpeed, -255, 255);

rightSpeed = constrain(rightSpeed, -255, 255);

if (leftSpeed >= 0) {

digitalWrite(L_IN1, LOW); digitalWrite(L_IN2, HIGH); analogWrite(L_PWM, leftSpeed);

} else {

digitalWrite(L_IN1, HIGH); digitalWrite(L_IN2, LOW); analogWrite(L_PWM, abs(leftSpeed));

}

if (rightSpeed >= 0) {

digitalWrite(R_IN1, LOW); digitalWrite(R_IN2, HIGH); analogWrite(R_PWM, rightSpeed);

} else {

digitalWrite(R_IN1, HIGH); digitalWrite(R_IN2, LOW); analogWrite(R_PWM, abs(rightSpeed));

}

}

void stopMotors() {

analogWrite(L_PWM, 0); analogWrite(R_PWM, 0);

digitalWrite(L_IN1, LOW); digitalWrite(L_IN2, LOW);

digitalWrite(R_IN1, LOW); digitalWrite(R_IN2, LOW);

}

# 3.Tunnel code

/*

* ChromoBot - Tunnel Only (Strict AND Logic)

* ------------------------------------------------

* PINS: Left(7,8) | Right(11,12)

* RANGE: 6cm - 20cm

* BEHAVIOR:

* - Robot moves ONLY if BOTH walls are detected.

* - If one wall is missing/far (>20cm), Robot STOPS.

* - If one wall is too close (<6cm), Robot STOPS.

*/

// --- MOTOR PINS ---

const int L_PWM = 2; const int L_IN1 = 22; const int L_IN2 = 23;

const int R_PWM = 3; const int R_IN1 = 24; const int R_IN2 = 25;

// --- ULTRASONIC PINS (Corrected) ---

const int TRIG_L = 7;  const int ECHO_L = 8;

const int TRIG_R = 11; const int ECHO_R = 12;

// --- TUNING ---

#define BASE_SPEED  60

#define WALL_KP     2     // Sensitivity

// --- STRICT RANGES ---

#define MIN_WALL_DIST 6

#define MAX_WALL_DIST 20

void setup() {

// Motor Setup

pinMode(L_PWM, OUTPUT); pinMode(L_IN1, OUTPUT); pinMode(L_IN2, OUTPUT);

pinMode(R_PWM, OUTPUT); pinMode(R_IN1, OUTPUT); pinMode(R_IN2, OUTPUT);

// Sensor Setup

pinMode(TRIG_L, OUTPUT); pinMode(ECHO_L, INPUT);

pinMode(TRIG_R, OUTPUT); pinMode(ECHO_R, INPUT);

Serial.begin(9600);

Serial.println("--- STRICT TUNNEL MODE START ---");

delay(2000);

}

void loop() {

// 1. READ SENSORS

int distLeft  = getDistance(TRIG_L, ECHO_L);

int distRight = getDistance(TRIG_R, ECHO_R);

Serial.print("L: "); Serial.print(distLeft);

Serial.print(" | R: "); Serial.println(distRight);

// 2. CHECK VALIDITY

bool validLeft  = (distLeft >= MIN_WALL_DIST && distLeft <= MAX_WALL_DIST);

bool validRight = (distRight >= MIN_WALL_DIST && distRight <= MAX_WALL_DIST);

// 3. STRICT DECISION (AND Logic)

if (validLeft && validRight) {

// CONDITION: BOTH sensors are safe.

// ACTION: Calculate error and move.

int error = distLeft - distRight;

int correction = error * WALL_KP;

move(BASE_SPEED - correction, BASE_SPEED + correction);

}

else {

// CONDITION: One or Both sensors are unsafe.

// ACTION: Immediate Stop.

stopMotors();

}

delay(50); // Loop stability

}

// --- HELPERS ---

int getDistance(int trig, int echo) {

digitalWrite(trig, LOW); delayMicroseconds(2);

digitalWrite(trig, HIGH); delayMicroseconds(10);

digitalWrite(trig, LOW);

long duration = pulseIn(echo, HIGH, 30000);

if (duration == 0) return 999;

return duration * 0.0343 / 2;

}

void move(int leftSpeed, int rightSpeed) {

leftSpeed = constrain(leftSpeed, 0, 120);

rightSpeed = constrain(rightSpeed, 0, 120);

if (leftSpeed > 0) {

digitalWrite(L_IN1, LOW); digitalWrite(L_IN2, HIGH); analogWrite(L_PWM, leftSpeed);

} else {

digitalWrite(L_IN1, LOW); digitalWrite(L_IN2, LOW); analogWrite(L_PWM, 0);

}

if (rightSpeed > 0) {

digitalWrite(R_IN1, LOW); digitalWrite(R_IN2, HIGH); analogWrite(R_PWM, rightSpeed);

} else {

digitalWrite(R_IN1, LOW); digitalWrite(R_IN2, LOW); analogWrite(R_PWM, 0);

}

}

void stopMotors() {

analogWrite(L_PWM, 0); analogWrite(R_PWM, 0);

digitalWrite(L_IN1, LOW); digitalWrite(L_IN2, LOW);

digitalWrite(R_IN1, LOW); digitalWrite(R_IN2, LOW);

}

Color sensor code

#include <Wire.h>

#include "Adafruit_TCS34725.h"

Adafruit_TCS34725 tcs = Adafruit_TCS34725(TCS34725_INTEGRATIONTIME_50MS, TCS34725_GAIN_1X);

void setup() {

Serial.begin(9600);

if (!tcs.begin()) {

Serial.println("No TCS34725 found ... check your connections");

while (1);

}

}

void loop() {

float r, g, b;

// This function gets the RGB data tailored for standard RGB (0-255)

tcs.getRGB(&r, &g, &b);

Serial.print("R: "); Serial.print((int)r); Serial.print(" ");

Serial.print("G: "); Serial.print((int)g); Serial.print(" ");

Serial.print("B: "); Serial.print((int)b); Serial.print(" ");

// Simple logic test

if ((int)r > (int)g && (int)r > (int)b) {

Serial.println("-> Detected: RED");

}

else if ((int)g > (int)r && (int)g > (int)b) {

Serial.println("-> Detected: GREEN");

}

else if ((int)b > (int)r && (int)b > (int)g) {

Serial.println("-> Detected: BLUE");

}

else {

Serial.println("-> Ambiguous");

}

delay(100);

}

# 4.Merged final (Line following + tunnel)

/*

* ChromoBot - MERGED FINAL

* ------------------------------------------------

* PHASE 1: White Line Following (with Bend/Drift Logic)

* PHASE 2: Strict Tunnel Navigation (Starts when Line is lost/Black Floor detected)

* * PINS:

* - Motors: Left(2, 22, 23) | Right(3, 24, 25)

* - IR Sensors: 44, 45, 46, 47, 48

* - Ultrasonic: Left(7,8) | Right(11,12)

*/

// ==========================================

//               PIN DEFINITIONS

// ==========================================

// --- MOTORS ---

const int L_PWM = 2; const int L_IN1 = 22; const int L_IN2 = 23;

const int R_PWM = 3; const int R_IN1 = 24; const int R_IN2 = 25;

// --- IR SENSORS (Line) ---

const int S1 = 44; const int S2 = 45; const int S3 = 46; const int S4 = 47; const int S5 = 48;

// --- ULTRASONIC SENSORS (Tunnel) ---

const int TRIG_L = 7;  const int ECHO_L = 8;

const int TRIG_R = 11; const int ECHO_R = 12;

// ==========================================

//               TUNING VARS

// ==========================================

// --- LINE FOLLOWER TUNING ---

#define PID_SPEED   60

#define ALIGN_SPEED 60

#define TURN_SPEED  100

#define DRIFT_LIMIT 400     // Max time to rotate for drift (ms)

#define ALIGN_TIME  1200    // 1.2 Seconds Blind Forward

#define KP 18.0

#define KD 22.0

// --- TUNNEL TUNING ---

#define TUNNEL_BASE_SPEED  60

#define WALL_KP            2      // Sensitivity

#define MIN_WALL_DIST      6

#define MAX_WALL_DIST      20

// --- LOGIC MAP ---

#define LINE 1   // White

#define FLOOR 0  // Black

// --- GLOBALS ---

int lastError = 0;

bool isRunning = true; // Main Line Follower Flag

void setup() {

// Motor Pins

pinMode(L_PWM, OUTPUT); pinMode(L_IN1, OUTPUT); pinMode(L_IN2, OUTPUT);

pinMode(R_PWM, OUTPUT); pinMode(R_IN1, OUTPUT); pinMode(R_IN2, OUTPUT);

// IR Pins

pinMode(S1, INPUT); pinMode(S2, INPUT); pinMode(S3, INPUT); pinMode(S4, INPUT); pinMode(S5, INPUT);

// Ultrasonic Pins

pinMode(TRIG_L, OUTPUT); pinMode(ECHO_L, INPUT);

pinMode(TRIG_R, OUTPUT); pinMode(ECHO_R, INPUT);

Serial.begin(9600);

Serial.println("--- READY: LINE MODE -> TUNNEL MODE ---");

delay(2000);

}

// ==========================================

//               MAIN LOOP

// ==========================================

void loop() {

if (!isRunning) return; // If we finished or stopped, do nothing.

int s1 = digitalRead(S1);

int s2 = digitalRead(S2);

int s3 = digitalRead(S3);

int s4 = digitalRead(S4);

int s5 = digitalRead(S5);

int activeSensors = s1 + s2 + s3 + s4 + s5;

// ----------------------------------------

// TRANSITION: CHECK FOR LINE LOSS (All Black)

// ----------------------------------------

if (activeSensors == 0) {

delay(100); // 100ms Debounce

if (digitalRead(S3) == FLOOR) {

Serial.println("--- LINE LOST: SWITCHING TO TUNNEL ---");

stopMotors();

delay(1000);       // Brief pause before tunnel logic starts

navigateTunnel();  // <--- JUMPS TO TUNNEL LOGIC FOREVER

isRunning = false; // Effectively ends the main loop

return;

}

}

// ----------------------------------------

// PHASE 1: LINE FOLLOWING LOGIC

// ----------------------------------------

// PRIORITY 1: BEND DETECTION

if (activeSensors >= 4) {

if (s1 == LINE && s2 == LINE && s3 == LINE && s4 == LINE) {

handleBend(-1); return;

}

if (s2 == LINE && s3 == LINE && s4 == LINE && s5 == LINE) {

handleBend(1); return;

}

}

// PRIORITY 2: DRIFT DETECTION

if (activeSensors == 3) {

if (s1 == LINE && s2 == LINE && s3 == LINE) {

handleDrift(-1); return;

}

if (s3 == LINE && s4 == LINE && s5 == LINE) {

handleDrift(1); return;

}

}

// PRIORITY 3: NORMAL PID

if (activeSensors > 0) {

int error = (s1 * -4) + (s2 * -2) + (s3 * 0) + (s4 * 2) + (s5 * 4);

error = error / activeSensors;

int P = error;

int D = error - lastError;

int correction = (KP * P) + (KD * D);

lastError = error;

move(PID_SPEED + correction, PID_SPEED - correction);

}

}

// ==========================================

//             PHASE 2: TUNNEL LOGIC

// ==========================================

void navigateTunnel() {

Serial.println("--- TUNNEL MODE ACTIVE ---");

// Locked Loop: Robot stays here forever (or until restart)

while (true) {

// 1. READ SENSORS

int distLeft  = getDistance(TRIG_L, ECHO_L);

int distRight = getDistance(TRIG_R, ECHO_R);

// 2. CHECK VALIDITY (Strict 10-20cm logic)

bool validLeft  = (distLeft >= MIN_WALL_DIST && distLeft <= MAX_WALL_DIST);

bool validRight = (distRight >= MIN_WALL_DIST && distRight <= MAX_WALL_DIST);

// 3. STRICT DECISION (AND Logic)

if (validLeft && validRight) {

// BOTH SAFE -> MOVE

int error = distLeft - distRight;

int correction = error * WALL_KP;

int leftSpeed = TUNNEL_BASE_SPEED - correction;

int rightSpeed = TUNNEL_BASE_SPEED + correction;

// CLAMP: Mimic the strict tunnel code (Don't reverse, just stop wheel)

if (leftSpeed < 0) leftSpeed = 0;

if (rightSpeed < 0) rightSpeed = 0;

// Clamp Max

if (leftSpeed > 120) leftSpeed = 120;

if (rightSpeed > 120) rightSpeed = 120;

move(leftSpeed, rightSpeed);

} else {

// ONE OR BOTH UNSAFE -> STOP

stopMotors();

// Robot waits here until walls are valid again (e.g., if you manually reset it)

// or it just stays stopped as requested.

}

delay(50); // Loop stability

}

}

// ==========================================

//               HELPERS

// ==========================================

int getDistance(int trig, int echo) {

digitalWrite(trig, LOW); delayMicroseconds(2);

digitalWrite(trig, HIGH); delayMicroseconds(10);

digitalWrite(trig, LOW);

long duration = pulseIn(echo, HIGH, 30000);

if (duration == 0) return 999;

return duration * 0.0343 / 2;

}

void handleBend(int direction) {

stopMotors(); delay(1000);

move(ALIGN_SPEED, ALIGN_SPEED); delay(ALIGN_TIME);

stopMotors(); delay(1000);

spinUntilMiddle3(direction, 6000);

}

void handleDrift(int direction) {

stopMotors(); delay(1000);

spinUntilMiddle3(direction, DRIFT_LIMIT);

}

void spinUntilMiddle3(int direction, int timeoutMs) {

unsigned long startTime = millis();

if (direction == -1) {

digitalWrite(L_IN1, LOW); digitalWrite(L_IN2, HIGH); analogWrite(L_PWM, TURN_SPEED);

digitalWrite(R_IN1, HIGH); digitalWrite(R_IN2, LOW); analogWrite(R_PWM, TURN_SPEED);

} else {

digitalWrite(L_IN1, HIGH); digitalWrite(L_IN2, LOW); analogWrite(L_PWM, TURN_SPEED);

digitalWrite(R_IN1, LOW); digitalWrite(R_IN2, HIGH); analogWrite(R_PWM, TURN_SPEED);

}

delay(100);

while (millis() - startTime < timeoutMs) {

int s2 = digitalRead(S2);

int s3 = digitalRead(S3);

int s4 = digitalRead(S4);

if (s2 == LINE && s3 == LINE && s4 == LINE) {

stopMotors(); delay(200); return;

}

}

stopMotors();

}

// --- GLOBAL MOTOR DRIVER ---

void move(int leftSpeed, int rightSpeed) {

// Constrain generally to PWM limits

leftSpeed = constrain(leftSpeed, -255, 255);

rightSpeed = constrain(rightSpeed, -255, 255);

if (leftSpeed >= 0) {

digitalWrite(L_IN1, LOW); digitalWrite(L_IN2, HIGH); analogWrite(L_PWM, leftSpeed);

} else {

digitalWrite(L_IN1, HIGH); digitalWrite(L_IN2, LOW); analogWrite(L_PWM, abs(leftSpeed));

}

if (rightSpeed >= 0) {

digitalWrite(R_IN1, LOW); digitalWrite(R_IN2, HIGH); analogWrite(R_PWM, rightSpeed);

} else {

digitalWrite(R_IN1, HIGH); digitalWrite(R_IN2, LOW); analogWrite(R_PWM, abs(rightSpeed));

}

}

void stopMotors() {

analogWrite(L_PWM, 0); analogWrite(R_PWM, 0);

digitalWrite(L_IN1, LOW); digitalWrite(L_IN2, LOW);

digitalWrite(R_IN1, LOW); digitalWrite(R_IN2, LOW);

}

# 5.Final (without color sorting)

/*

ChromoBot - STRICT LOGIC (CORRECTED)

Deterministic state machine implementation

*/

enum RobotState {

LINE_FOLLOW,

TUNNEL,

POST_TUNNEL_ALIGN,

SEQUENCE,

FINAL_STOP

};

RobotState state = LINE_FOLLOW;

// ================= PINS =================

const int L_PWM = 2; const int L_IN1 = 22; const int L_IN2 = 23;

const int R_PWM = 3; const int R_IN1 = 24; const int R_IN2 = 25;

const int S1 = 44; const int S2 = 45; const int S3 = 46; const int S4 = 47; const int S5 = 48;

const int TRIG_L = 7; const int ECHO_L = 8;

const int TRIG_R = 11; const int ECHO_R = 12;

const int TRIG_MID = 10; const int ECHO_MID = 9;

// ================= TUNING =================

#define PID_SPEED 60

#define TURN_SPEED 100

#define ALIGN_SPEED 60

#define ALIGN_TIME 350

#define KP 18.0

#define KD 22.0

#define TUNNEL_BASE_SPEED 60

#define WALL_KP 3

#define PICKUP_MIN_DIST 10

#define PICKUP_MAX_DIST 20

#define LINE 1

#define FLOOR 0

// ================= GLOBALS =================

int lastError = 0;

int tJunctionsPassed = 0;

bool tunnelFinished = false;

// ==========================================

void setup() {

pinMode(L_PWM, OUTPUT); pinMode(L_IN1, OUTPUT); pinMode(L_IN2, OUTPUT);

pinMode(R_PWM, OUTPUT); pinMode(R_IN1, OUTPUT); pinMode(R_IN2, OUTPUT);

pinMode(S1, INPUT); pinMode(S2, INPUT); pinMode(S3, INPUT); pinMode(S4, INPUT); pinMode(S5, INPUT);

pinMode(TRIG_L, OUTPUT); pinMode(ECHO_L, INPUT);

pinMode(TRIG_R, OUTPUT); pinMode(ECHO_R, INPUT);

pinMode(TRIG_MID, OUTPUT); pinMode(ECHO_MID, INPUT);

}

void loop() {

if (state == FINAL_STOP) return;

int s1 = digitalRead(S1);

int s2 = digitalRead(S2);

int s3 = digitalRead(S3);

int s4 = digitalRead(S4);

int s5 = digitalRead(S5);

int active = s1 + s2 + s3 + s4 + s5;

// ================= FINAL PICKUP =================

if (state == SEQUENCE && tJunctionsPassed == 2) {

int d = getDistance(TRIG_MID, ECHO_MID);

if (d >= PICKUP_MIN_DIST && d <= PICKUP_MAX_DIST) {

stopMotors();

state = FINAL_STOP;

return;

}

}

// ================= LINE LOSS → TUNNEL =================

if (state == LINE_FOLLOW && active == 0) {

delay(50);

if (digitalRead(S3) == FLOOR) {

stopMotors();

delay(300);

state = TUNNEL;

return;

}

}

// ================= TUNNEL MODE =================

if (state == TUNNEL) {

if (s1 == LINE || s3 == LINE || s5 == LINE) {

stopMotors();

state = POST_TUNNEL_ALIGN;

return;

}

int dl = getDistance(TRIG_L, ECHO_L);

int dr = getDistance(TRIG_R, ECHO_R);

int error = dl - dr;

int corr = error * WALL_KP;

move(TUNNEL_BASE_SPEED - corr, TUNNEL_BASE_SPEED + corr);

delay(40);

return;

}

// ================= POST TUNNEL ALIGN =================

if (state == POST_TUNNEL_ALIGN) {

if (s2 && s3 && s4) {

tunnelFinished = true;

lastError = 0;

state = SEQUENCE;

return;

}

if (s1 || s2) spinUntilMiddle3(-1);

else if (s4 || s5) spinUntilMiddle3(1);

if (digitalRead(S2) && digitalRead(S3) && digitalRead(S4)) {

tunnelFinished = true;

lastError = 0;

state = SEQUENCE;

}

return;

}

// ================= INTERSECTION DETECTION =================

if ((active >= 4) ||

(s1 && s2) ||

(s4 && s5)) {

move(PID_SPEED, PID_SPEED);

delay(50);

stopMotors();

int r1 = digitalRead(S1);

int r2 = digitalRead(S2);

int r3 = digitalRead(S3);

int r4 = digitalRead(S4);

int r5 = digitalRead(S5);

int rActive = r1 + r2 + r3 + r4 + r5;

bool confirmed =

(rActive >= 4) ||

(r1 && r2) ||

(r4 && r5);

if (confirmed) {

if (state == SEQUENCE) {

if (tJunctionsPassed < 2) {

executeLeftTurn();

tJunctionsPassed++;

}

} else {

if (r1 && r2) executeLeftTurn();

else if (r4 && r5) executeRightTurn();

}

return;

}

}

// ================= PID LINE FOLLOW =================

if (active > 0) {

int error = (s1 * -4) + (s2 * -2) + (s3 * 0) + (s4 * 2) + (s5 * 4);

error /= active;

int P = error;

int D = error - lastError;

int corr = (KP * P) + (KD * D);

lastError = error;

move(PID_SPEED + corr, PID_SPEED - corr);

}

}

// ==========================================

// TURN SEQUENCES

// ==========================================

void executeLeftTurn() {

stopMotors();

delay(200);

move(ALIGN_SPEED, ALIGN_SPEED);

delay(ALIGN_TIME);

stopMotors();

spinUntilMiddle3(-1);

}

void executeRightTurn() {

stopMotors();

delay(200);

move(ALIGN_SPEED, ALIGN_SPEED);

delay(ALIGN_TIME);

stopMotors();

spinUntilMiddle3(1);

}

void spinUntilMiddle3(int dir) {

if (dir == -1)

move(-TURN_SPEED, TURN_SPEED);

else

move(TURN_SPEED, -TURN_SPEED);

while (true) {

if (digitalRead(S2) && digitalRead(S3) && digitalRead(S4)) {

stopMotors();

delay(150);

return;

}

}

}

// ==========================================

int getDistance(int trig, int echo) {

digitalWrite(trig, LOW); delayMicroseconds(2);

digitalWrite(trig, HIGH); delayMicroseconds(10);

digitalWrite(trig, LOW);

long dur = pulseIn(echo, HIGH, 30000);

if (dur == 0) return 999;

return dur * 0.0343 / 2;

}

void move(int l, int r) {

l = constrain(l, -255, 255);

r = constrain(r, -255, 255);

if (l >= 0) { digitalWrite(L_IN1, LOW); digitalWrite(L_IN2, HIGH); }

else { digitalWrite(L_IN1, HIGH); digitalWrite(L_IN2, LOW); }

analogWrite(L_PWM, abs(l));

if (r >= 0) { digitalWrite(R_IN1, LOW); digitalWrite(R_IN2, HIGH); }

else { digitalWrite(R_IN1, HIGH); digitalWrite(R_IN2, LOW); }

analogWrite(R_PWM, abs(r));

}

void stopMotors() {

analogWrite(L_PWM, 0);

analogWrite(R_PWM, 0);

digitalWrite(L_IN1, LOW); digitalWrite(L_IN2, LOW);

digitalWrite(R_IN1, LOW); digitalWrite(R_IN2, LOW);

}

# 6. Final code

#include <Wire.h>

#include "Adafruit_TCS34725.h"

// ================= SETUP TCS34725 =================

// 50ms integration time, 1x gain for accurate close-range reading

Adafruit_TCS34725 tcs = Adafruit_TCS34725(TCS34725_INTEGRATIONTIME_50MS, TCS34725_GAIN_1X);

// ================= STATES =================

enum RobotState {

LINE_FOLLOW,          // Phase 1: Start -> Tunnel

TUNNEL,               // Phase 2: Wall Follow

POST_TUNNEL_ALIGN,    // Phase 3: Exit Alignment

SEQUENCE_BLUE,        // Phase 3: Navigate to Blue Tower

COLOR_CHECK_BLUE,     // Phase 4: Identify Blue

TO_DROP_BLUE,         // Phase 5: Go to Drop Point (Ignore Right)

DROP_ACTION_BLUE,     // Phase 5: Drop Logic

TO_GREEN_TOWER,       // Phase 6: Navigate to Green (Left -> Ignore Right -> Right)

COLOR_CHECK_GREEN,    // Phase 6: Identify Green

TO_DROP_GREEN,        // Phase 7: Go to Drop Point (Ignore Left)

DROP_ACTION_GREEN,    // Phase 7: Drop Logic

MISSION_COMPLETE      // Stop

};

RobotState state = LINE_FOLLOW;

// ================= PINS =================

// MOTORS

const int L_PWM = 2; const int L_IN1 = 22; const int L_IN2 = 23;

const int R_PWM = 3; const int R_IN1 = 24; const int R_IN2 = 25;

// IR SENSORS (Left to Right)

const int S1 = 44; const int S2 = 45; const int S3 = 46; const int S4 = 47; const int S5 = 48;

// ULTRASONICS

const int TRIG_L = 7;   const int ECHO_L = 8;

const int TRIG_R = 11;  const int ECHO_R = 12;

const int TRIG_MID = 10; const int ECHO_MID = 9;

// ================= TUNING =================

#define PID_SPEED   60

#define TURN_SPEED  100

#define ALIGN_SPEED 60

// TIMERS (Adjust these based on robot speed/battery)

#define ALIGN_TIME  350     // Time to drive STRAIGHT before turning

#define BLIND_TIME  400     // Time to ignore junctions (Loophole Fix)

#define UTURN_TIME  800     // Blind spin time for U-Turn

// PID

#define KP 18.0

#define KD 22.0

// TUNNEL

#define TUNNEL_BASE_SPEED 60

#define WALL_KP 3

// DISTANCES

#define PICKUP_MIN 8

#define PICKUP_MAX 20

// LOGIC

#define LINE 1

#define FLOOR 0

// ================= GLOBALS =================

int lastError = 0;

int tJunctionsPassed = 0; // Re-used counter for phases

bool tunnelFinished = false;

// ==========================================

void setup() {

Serial.begin(9600);

// Motor Pins

pinMode(L_PWM, OUTPUT); pinMode(L_IN1, OUTPUT); pinMode(L_IN2, OUTPUT);

pinMode(R_PWM, OUTPUT); pinMode(R_IN1, OUTPUT); pinMode(R_IN2, OUTPUT);

// Sensor Pins

pinMode(S1, INPUT); pinMode(S2, INPUT); pinMode(S3, INPUT); pinMode(S4, INPUT); pinMode(S5, INPUT);

pinMode(TRIG_L, OUTPUT); pinMode(ECHO_L, INPUT);

pinMode(TRIG_R, OUTPUT); pinMode(ECHO_R, INPUT);

pinMode(TRIG_MID, OUTPUT); pinMode(ECHO_MID, INPUT);

// Color Sensor Init

if (tcs.begin()) {

Serial.println("Sensor Found");

} else {

Serial.println("No TCS34725 found ... check connections");

while (1); // Halt

}

Serial.println("--- MISSION START ---");

delay(2000);

}

void loop() {

if (state == MISSION_COMPLETE) {

stopMotors();

return;

}

// READ SENSORS

int s1 = digitalRead(S1); int s2 = digitalRead(S2);

int s3 = digitalRead(S3); int s4 = digitalRead(S4); int s5 = digitalRead(S5);

int active = s1 + s2 + s3 + s4 + s5;

// ============================================

//             PHASE 7: GREEN DROP

// ============================================

if (state == TO_DROP_GREEN) {

// 1. ARRIVAL: Line Ends (All Black)

if (active == 0) {

// Double check to ensure it's not a gap

delay(50);

if (digitalRead(S3) == FLOOR) {

// Logic says active 0 is black line, but confirm it's not a hole

// Assuming White Line on Black Floor: 0 active means NO LINE (Black).

stopMotors();

state = DROP_ACTION_GREEN;

return;

}

}

// 2. LOOPHOLE FIX: Ignore LEFT Turns (Passing 3rd T-Junc)

// If we see Left Wing (S1+S2) or Junction (4+)

if (active >= 4 || (s1 && s2)) {

Serial.println("-> Ignoring Left Turn (Blind Forward)");

ignoreTurn();

return;

}

runPID(s1, s2, s3, s4, s5, active);

return;

}

if (state == DROP_ACTION_GREEN) {

Serial.println("--- DROPPING GREEN CUBE ---");

delay(1000); // Simulate Drop

state = MISSION_COMPLETE;

return;

}

// ============================================

//             PHASE 6: TO GREEN TOWER

// ============================================

if (state == TO_GREEN_TOWER) {

// Check Ultrasonic for Arrival (Only valid after the final Right turn)

if (tJunctionsPassed == 3) {

int d = getDistance(TRIG_MID, ECHO_MID);

if (d >= PICKUP_MIN && d <= PICKUP_MAX) {

stopMotors();

state = COLOR_CHECK_GREEN;

return;

}

}

// Intersection Handling

if (active >= 4 || (s1&&s2) || (s4&&s5)) {

if (verifyIntersection()) {

// Turn 1: 1st T-Junc -> LEFT

if (tJunctionsPassed == 0) {

executeTurn(-1); // Left

tJunctionsPassed++;

return;

}

// Loophole: Passing Tunnel T (Right Side) -> IGNORE

if (tJunctionsPassed == 1) {

Serial.println("-> Crossing Tunnel T (Blind Forward)");

ignoreTurn();

tJunctionsPassed++; // Increment to signify we passed it

return;

}

// Turn 2: 3rd T-Junc -> RIGHT

if (tJunctionsPassed == 2) {

executeTurn(1); // Right

tJunctionsPassed++;

return;

}

}

}

runPID(s1, s2, s3, s4, s5, active);

return;

}

if (state == COLOR_CHECK_GREEN) {

checkColorAverage(false); // Check Green

performUTurn();

state = TO_DROP_GREEN;

return;

}

// ============================================

//             PHASE 5: TO BLUE DROP

// ============================================

if (state == TO_DROP_BLUE) {

// 1. ARRIVAL: Line Ends

if (active == 0) {

delay(50);

// Assuming active 0 = All Black (Drop Point)

stopMotors();

state = DROP_ACTION_BLUE;

return;

}

// 2. LOOPHOLE FIX: Ignore RIGHT Turns (Passing 1st T-Junc)

if (active >= 4 || (s4 && s5)) {

Serial.println("-> Ignoring Right Turn (Blind Forward)");

ignoreTurn();

return;

}

runPID(s1, s2, s3, s4, s5, active);

return;

}

if (state == DROP_ACTION_BLUE) {

Serial.println("--- DROPPING BLUE CUBE ---");

delay(1000);

performUTurn();

tJunctionsPassed = 0; // Reset counter for Phase 6

state = TO_GREEN_TOWER;

return;

}

// ============================================

//             PHASE 4: COLOR CHECK 1

// ============================================

if (state == COLOR_CHECK_BLUE) {

checkColorAverage(true); // Check Blue

performUTurn();

state = TO_DROP_BLUE;

return;

}

// ============================================

//             PHASE 3: SEQUENCE BLUE

// ============================================

if (state == SEQUENCE_BLUE) {

// Check Arrival

if (tJunctionsPassed >= 2) {

int d = getDistance(TRIG_MID, ECHO_MID);

if (d >= PICKUP_MIN && d <= PICKUP_MAX) {

stopMotors();

state = COLOR_CHECK_BLUE;

return;

}

}

// Check Turns

if (active >= 4 || (s1&&s2) || (s4&&s5)) {

if (verifyIntersection()) {

if (tJunctionsPassed < 2) {

executeTurn(-1); // Force Left x2

tJunctionsPassed++;

}

}

}

runPID(s1, s2, s3, s4, s5, active);

return;

}

// ============================================

//             PHASE 2: TUNNEL EXIT

// ============================================

if (state == POST_TUNNEL_ALIGN) {

// Auto-Balance: Scan if we are crooked

if (s2 && s3 && s4) {

stopMotors(); delay(200);

state = SEQUENCE_BLUE;

tJunctionsPassed = 0;

return;

}

if (s1 || s2) spinUntilMiddle3(-1); // Spin Left

else if (s4 || s5) spinUntilMiddle3(1); // Spin Right

// Failsafe: if s3 only is active, we are good

if (s3) {

state = SEQUENCE_BLUE;

tJunctionsPassed = 0;

}

return;

}

if (state == TUNNEL) {

// Exit Condition: Any Line Detected

if (s1 == LINE || s3 == LINE || s5 == LINE) {

stopMotors();

delay(500); // Settle

state = POST_TUNNEL_ALIGN;

return;

}

// Wall Follow

int dl = getDistance(TRIG_L, ECHO_L);

int dr = getDistance(TRIG_R, ECHO_R);

int error = dl - dr;

int corr = error * WALL_KP;

move(TUNNEL_BASE_SPEED - corr, TUNNEL_BASE_SPEED + corr);

delay(40);

return;

}

// ============================================

//             PHASE 1: LINE FOLLOW

// ============================================

if (state == LINE_FOLLOW) {

// Check Tunnel Entry

if (active == 0) {

delay(50);

if (digitalRead(S3) == FLOOR) { // Confirm All Black + Floor Ground

stopMotors(); delay(500);

state = TUNNEL;

return;

}

}

// Phase 1 Turns (Standard)

if (active >= 4 || (s1&&s2) || (s4&&s5)) {

if (verifyIntersection()) {

if (s1 && s2) executeTurn(-1);      // Left

else if (s4 && s5) executeTurn(1);  // Right

}

}

runPID(s1, s2, s3, s4, s5, active);

}

}

// ==========================================

//             HELPER FUNCTIONS

// ==========================================

// ROBUST COLOR CHECKER

void checkColorAverage(bool lookForBlue) {

long rSum = 0, gSum = 0, bSum = 0;

// Take 5 samples

for(int i=0; i<5; i++) {

float r, g, b;

tcs.getRGB(&r, &g, &b);

rSum += (long)r; gSum += (long)g; bSum += (long)b;

delay(20);

}

long r = rSum/5; long g = gSum/5; long b = bSum/5;

Serial.print("Avg Color: R"); Serial.print(r); Serial.print(" G"); Serial.print(g); Serial.print(" B"); Serial.println(b);

// Simple Ratio Logic (Adjust 1.2 multiplier if needed)

bool isBlue  = (b > r * 1.2 && b > g * 1.2);

bool isGreen = (g > r * 1.2 && g > b * 1.2);

if (lookForBlue && isBlue) {

Serial.println("-> BLUE CONFIRMED: Picking Up...");

delay(2000); // Simulate Arm

}

else if (!lookForBlue && isGreen) {

Serial.println("-> GREEN CONFIRMED: Picking Up...");

delay(2000); // Simulate Arm

}

else {

Serial.println("-> Color Mismatch or Ambiguous");

delay(500);

}

}

void performUTurn() {

Serial.println("-> U-TURN");

stopMotors(); delay(500);

move(TURN_SPEED, -TURN_SPEED); // Spin

delay(UTURN_TIME); // Blind Spin to clear line

spinUntilMiddle3(1); // Seek Line (Spin Right)

}

void ignoreTurn() {

// Blindly move forward to clear the intersection

move(PID_SPEED, PID_SPEED);

delay(BLIND_TIME);

}

bool verifyIntersection() {

move(PID_SPEED, PID_SPEED);

delay(50); // Move into the intersection slightly

stopMotors();

int r1 = digitalRead(S1); int r2 = digitalRead(S2);

int r3 = digitalRead(S3); int r4 = digitalRead(S4); int r5 = digitalRead(S5);

int active = r1 + r2 + r3 + r4 + r5;

return (active >= 3);

}

void executeTurn(int dir) {

stopMotors(); delay(200);

move(ALIGN_SPEED, ALIGN_SPEED);

delay(ALIGN_TIME); // Align wheels with center of turn

stopMotors();

spinUntilMiddle3(dir);

}

void spinUntilMiddle3(int dir) {

if (dir == -1) move(-TURN_SPEED, TURN_SPEED); // Left

else move(TURN_SPEED, -TURN_SPEED); // Right

// Safety timeout to prevent infinite loop

unsigned long start = millis();

while (millis() - start < 5000) {

if (digitalRead(S2) && digitalRead(S3) && digitalRead(S4)) {

stopMotors(); delay(150); return;

}

}

stopMotors(); // Timeout stop

}

void runPID(int s1, int s2, int s3, int s4, int s5, int active) {

if (active == 0) return;

int error = (s1 * -4) + (s2 * -2) + (s3 * 0) + (s4 * 2) + (s5 * 4);

error /= active;

int P = error;

int D = error - lastError;

int corr = (KP * P) + (KD * D);

lastError = error;

move(PID_SPEED + corr, PID_SPEED - corr);

}

void move(int l, int r) {

l = constrain(l, -255, 255); r = constrain(r, -255, 255);

if (l >= 0) { digitalWrite(L_IN1, LOW); digitalWrite(L_IN2, HIGH); }

else { digitalWrite(L_IN1, HIGH); digitalWrite(L_IN2, LOW); }

analogWrite(L_PWM, abs(l));

if (r >= 0) { digitalWrite(R_IN1, LOW); digitalWrite(R_IN2, HIGH); }

else { digitalWrite(R_IN1, HIGH); digitalWrite(R_IN2, LOW); }

analogWrite(R_PWM, abs(r));

}

void stopMotors() {

analogWrite(L_PWM, 0); analogWrite(R_PWM, 0);

digitalWrite(L_IN1, LOW); digitalWrite(L_IN2, LOW);

digitalWrite(R_IN1, LOW); digitalWrite(R_IN2, LOW);

}

int getDistance(int trig, int echo) {

digitalWrite(trig, LOW); delayMicroseconds(2);

digitalWrite(trig, HIGH); delayMicroseconds(10);

digitalWrite(trig, LOW);

long dur = pulseIn(echo, HIGH, 30000);

if (dur == 0) return 999;

return dur * 0.0343 / 2;

}